U N I C O M

===========

Terminal program for DOS

UC.EXE      Terminal program for DOS, COM1 to COM4,

with protocol analysis function for two ports

The menu language of Unicom is either English or German,

depending on the COUNTRY parameter in CONFIG.SYS. German

is selected for country=49. We recommend to use code page

437 (not 850) in order to display all graphic symbols

correctly.

Online help and mouse

---------------------

You will get a context-sensitive help by pressing ALT-F1

in any menu or in the main screen, or by clicking on the upper

right corner of the screen with the right mouse key.

The left mouse button or Enter accepts a selection, the right

mouse button or ESC aborts a menu or entry.

Configuration

-------------

First select a proper COM port and its settings in the

"configuration" menu. For COM3 or COM4 make sure to select

the correct address and interrupt. Do not use the same

interrupt for more than one COM port.

To start UC.EXE for a specific COM port, COM2 for instance,

enter UC /2 at the DOS command prompt.

Protocol analysis

-----------------

To watch the communication of two other sets, e.g. a computer

and a modem, you will need to make a special cable with four

connectors so that Unicom may watch the TXD data stream thru

RXD of its first port, and the RXD data thru RXD of its second

port. The data of each direction will be displayed in different

colors.

Upload and download

-------------------

Unicom supports Xmodem and Xmodem-1K for binary file transfers.

Select a block length of 128 for standard Xmodem or Xmodem-CRC,

and 1024 for Xmodem-1K.

Script files

------------

When sending a script file, some characters are interpreted:

<  (less than)         Hangup (DTR+RTS low for 2 seconds)

>  (greater than)      Wait for connect, i.e. until DCD=1

'  (apostrophe)        Wait until nothing received for 1 s

_  (underline)         Pause one second

#  (pound symbol)      Do not interpret the next character,

e.g. "#>" to send ">"

Example for a script file:

ATD 123456              Dials a number

>'''''HELP              Waits for connect, pauses 5 s and

then enters HELP

Macros

------

You may configure macros (sequences of key strokes) which can

be executed automatically by pressing F1 to F10.

Macros may be concatenated. If you program ALT-1 (1 is the

key above the Q key, not on the numeric keypad), the program

will execute the F1 macro next.

Copyright

---------

(c) Shamrock Software GmbH, http://www.shamrock.de

-------------------------------------------------------------------------

Downloaded from the freeware section of www.shamrock.de